Protocollo Operativo SYNC: Preprocessing -> Estrazione MU (OTBioLab+) -> Coerenza

1) Scopo

Questo documento descrive **solo** la pipeline SYNC, coerente con:

•	
      allineamento_eeg_emg_sync.m

•	
      coerenza/coerenza_motoria_source_batch.m (versione sync)

Obiettivi:

1	
      allineamento temporale EEG/EMG robusto con trigger;

2	
      creazione file OTB con soli segmenti di movimento;

3	
      estrazione Motor Units (MU) in OTBioLab+;

4	
      calcolo coerenza cortico-sinergica con le stesse regole della pipeline sync.

2) Input/Output SYNC

Input run:

•	
      EEG .mff

•	
      EMG .otb+

Output principali (per run):

•	
      *_aligned_otb.mff

•	
      *_aligned_otb_events.csv

•	
      report.json

•	
      *_movement_only.otb+

•	
      *_movement_segments.csv

Output batch sync:

•	
      allineamento: batch_otb_mff/output_matlab_sync

•	
      coerenza: coerenza/coerenza_motoria_source_sync

3) Logica trigger SYNC (allineamento)

Regole operative:

1	
      **Trial gating** su DIN4/DIN5 (DIN1 non e' gate di trial validita').

2	
      Sequenza logica trial: start < keyboard < beep < end e end < keyboard successivo.

3	
      **Pulizia trigger** su AUX (rimozione edge troppo ravvicinati) con controllo di plausibilita' numerica.

4	
      **Gap minimo keyboard->beep = 50 ms** per evitare coppie quasi simultanee.

5	
      **DIN1 denso (onda sync)**: usato solo per rifinitura del fit affine (a,b) dopo il fit principale DIN4/DIN5.

6	
      Guardrail qualità fit: run bloccato se fit instabile (es. drift/mad fuori soglia).

4) Segmentazione movimento per OTB

Il file *_movement_only.otb+ viene creato automaticamente nella sync.

Regole segmentazione:

•	
      primaria: beep -> start

•	
      fallback automatico: beep -> end se start non e' successivo a beep

Tracciabilita':

•	
      *_movement_segments.csv: trial, tempi MFF, tempi OTB, sample, durata

•	
      report.json: movement_only.rule_used, n_segments, duration_s, eventuale reason

5) Estrazione MU in OTBioLab+ (sync)

Workflow consigliato:

1	
      aprire *_movement_only.otb+;

2	
      selezionare griglia/canali del muscolo target;

3	
      Processing > Decomposition > Decomponi;

4	
      editing manuale IPT/IDR;

5	
      salvare solo MU approvate;

6	
      esportare firing samples e tracce MU (.mat/.csv).

Nota operativa:

•	
      preferire blocchi grouped ~20-100 s per stabilita' di decomposizione.

6) QC MU (raccomandato)

Accettazione MU:

1	
      PNR >= 30 dB (criterio forte);

2	
      spike train fisiologicamente plausibile;

3	
      rimozione MU con pattern artefattuali dopo editing;

4	
      registrare MU accettate/scartate per run.

7) Coerenza SYNC: parametri vincolanti

La coerenza sync deve restare coerente col codice:

1	
      trial da eventi ricostruiti: beep -> first start before next beep;

2	
      durata minima trial: 0.50 s;

3	
      preprocessing EMG: band-pass 20-450 Hz, notch 50 Hz + armoniche, rettifica, low-pass 10 Hz;

4	
      riallineamento EMG su asse EEG con (a,b) del report sync;

5	
      NMF sinergie: maxK=6, target VAF globale >= 0.90, VAF minimo canale >= 0.75;

6	
      coerenza: downsample interno ~250 Hz, finestra ~0.8 s, overlap 50%, nfft >= 512;

7	
      bande: beta 13-30 Hz, sotto-bande 13-20 e 20-30 Hz.

8) Source-space in SYNC

Ordine di priorita':

1	
      usare sources_eeg.mat se disponibile (source_net);

2	
      fallback scalp-proxy se source assente/non valido;

3	
      salvare sempre mode e stato source in summary/report.

9) Checklist SYNC run-by-run

1	
      report.json presente e senza errori;

2	
      logical.enabled = true;

3	
      movement_only.enabled = true;

4	
      movement_segments.csv presente;

5	
      trial_info.csv presente in output coerenza sync;

6	
      verificare eventuale beep_to_end_fallback nei run finali.

10) Methods (paper-ready, versione sync)

10.1 Alignment and synchronization refinement

EEG and HD-EMG were aligned through trigger-based affine mapping. Trial validity was constrained by DIN4/DIN5 temporal order, while DIN1 was excluded from trial acceptance. In the sync pipeline, dense DIN1 trains were optionally used only to refine affine clock-drift parameters after the DIN4/DIN5 fit.

10.2 Trigger cleaning and trial enforcement

Auxiliary trigger streams underwent minimum-gap cleaning and count plausibility checks before mapping. Valid trials were reconstructed under strict logical order (start < keyboard < beep < end) with a minimum keyboard-beep separation of 50 ms.

10.3 Movement-only HD-EMG export

Movement-only OTB files were generated by concatenating valid trial windows. Primary windows were defined as beep-to-start; when start did not follow beep, beep-to-end fallback was applied and explicitly logged.

10.4 MU decomposition

MU decomposition was performed in OTBioLab+ (Decomponi), followed by manual review and approval. Firing samples and MU tracks were exported for downstream analyses. Quality control prioritized high-confidence units (recommended PNR threshold >= 30 dB).

10.5 Cortico-synergy coherence

Coherence was computed on beep->start trials (minimum 0.5 s), after EMG envelope preprocessing and EMG-to-EEG temporal remapping with run-specific affine parameters. NMF extracted up to six synergies with VAF constraints; beta-band coupling (13-30 Hz) and related metrics (z-coherence, lag-optimized beta coherence, PSI, ROI state-dependent contrasts) were computed with fixed spectral settings.

11) Riferimenti principali

•	
      Holobar et al., CKC decomposition (Clin Neurophysiol 2009): https://pubmed.ncbi.nlm.nih.gov/19208498/

•	
      Holobar et al., accuratezza vs intramuscolare (IEEE TNSRE 2010): https://pubmed.ncbi.nlm.nih.gov/20144921/

•	
      Holobar et al., PNR e soglia 30 dB (J Neural Eng 2014): https://pubmed.ncbi.nlm.nih.gov/24654270/

•	
      Hug et al., affidabilita' inter-operatore (J Electromyogr Kinesiol 2021): https://pubmed.ncbi.nlm.nih.gov/33838590/

•	
      OTBioLab+ User Manual (Decomponi / Extract MU): https://otbioelettronica.it/wp-content/uploads/wpfd/preview_files/User-Manual-OTBiolab%2818ad9899f3f21a5a1583584d5f11c0c0%29.pdf

•	
      Aggiornamenti recenti su decomposizione in condizioni complesse (J Physiol 2025): https://pubmed.ncbi.nlm.nih.gov/40096591/